Stormwater Management Facility

Operation and Maintenance

(O & M) Manual

For

Site/Project Name

Site/Project Address

Site/Project Legal Description

Pueblo, Colorado

DATE

Prepared for:

Name

Address

City, State, Zip

Phone

Prepared by:

Name

Address

City, State, Zip

Phone

Email

Contents

Section 1:  Description of Stormwater Facilities	1

Section 1.1:  General Site Description	1

Section 1.2:  General Stormwater Management Description	1

Section 1.3:  Stormwater Facilities Site Plan	1

Section 1.4:  On-Site Stormwater Management Facilities	1

Section 2:  Stormwater Management Facility Operations & Maintenance (O&M) Manual	1

Section 2.1:  Compliance with Stormwater Facility Maintenance Requirements	1

Section 2.2:  Inspection & Maintenance – Annual Reporting	1

Section 2.3:  Preventative Measures to Reduce Maintenance Costs	2

Section 2.4:  Access and Easements	2

Section 2.5:  Safety	2

Section 2.6:  Field Inspection Equipment	3

Section 2.7:  Inspecting Permanent Stormwater Control Measure Facilities	3

Section 2.8:  Maintaining Stormwater Management Facilities	3

Section 3:  Standard Operating Procedures for Inspection and Maintenance for Detention Basins (PSCMs)	4

Section 3.1:  Background	5

Section 3.2:  Inspecting Detention Basins (PSCMs)	5

Section 3.3:  Inspection Forms	10

Section 3.4:  Maintaining Detention Basins (PSCMs)	10

Appendix A:  Stormwater Facility Maintenance Agreement(s) – template (Remove sheets and replace with signed a notarized agreement	16

Appendix B:  PSCM Owner Inspection Checklist and Maintenance Reporting Form	21

Appendix C:  Maintenance Site Plan – change appendix name to reflect sheet name	25

# Section 1:  Description of Stormwater Facilities

This section documents the description of the stormwater facilities.

Describe the stormwater facility here.

## Section 1.1:  General Site Description

The site is located at ADDRESS.  LEGAL DESCRIPTION.  ACREAGE.  ZONING.  Etc.

## Section 1.2:  General Stormwater Management Description

This section is used to list how all onsite stormwater is conveyed.

## Section 1.3:  Stormwater Facilities Site Plan

Inspection or maintenance personnel may utilize the Maintenance Site Plan located in Appendix C (verify this location and name of plan is correct) for locating the stormwater facilities within this development.

## Section 1.4:  On-Site Stormwater Management Facilities

List the different facilities for the site and where they are located.  If a stormwater management facility or water quality control feature is used that is not covered in the O & M template, you shall include spec sheets, maintenance guidelines, failure guidelines, etc.  Add additional sections and appendixes to this O & M Manual as necessary.

# Section 2:  Stormwater Management Facility Operations & Maintenance (O&M) Manual

This section documents the standards and requirements in the O & M Manual.

## Section 2.1:  Compliance with Stormwater Facility Maintenance Requirements

All property owners are responsible for ensuring that stormwater facilities installed on their property are properly maintained, and that they function as designed.  In some cases, this maintenance responsibility may be assigned to others through special agreements.  The maintenance responsibility for a stormwater facility may be designated on the subdivision plat, the site development plan, and/or within a maintenance agreement for the property.  Property owners should be aware of their responsibilities regarding stormwater facility maintenance.  Maintenance agreement(s) associated with this property are provided in Appendix A.

A maintenance agreement template is located in Appendix A.  Replace the template with a signed and notarized agreement. (remove this statement prior to printing/submitting)

## Section 2.2:  Inspection & Maintenance – Annual Reporting

Requirements for the inspection and maintenance of stormwater facilities, as well as reporting requirements, are included in this manual.

Verification that the Stormwater facilities have been properly inspected and maintained; submittal of the required PSCM Owner Inspection Checklist form should be provided to Pueblo County Department of Public Works (PCPW) on an annual basis.  The annual reporting form should be provided to PCPW prior to April 1st of each year.

A copy of the Permanent Stormwater Control Measure Owner Inspection Checklist and Maintenance Reporting Form is located in Appendix B (verify this location is correct).  The form should be reviewed and submitted by the property owner or property manager to PCPW prior to April 1st of each year.

## Section 2.3:  Preventative Measures to Reduce Maintenance Costs

The most effective way to maintain your water quality facility is to prevent the pollutants from entering the facility in the first place.  Common pollutants include sediment, trash & debris, chemicals, dog waste, runoff from stored materials, illicit discharges into the storm drainage system and many others.  A complete maintenance program will include measures to address these potential contaminants, which will save time and money in the end.

Key points to consider in your maintenance program include:

- Educate property owners/residents to be aware of how their actions affect water quality and how they can help reduce maintenance costs.

- Keep properties, streets and gutters, and parking lots free of trash, debris, and lawn clippings.

- Ensure the proper disposal of hazardous wastes and chemicals.

- Plan lawn care to minimize the use of chemicals and pesticides.

- Sweep paved surfaces and put the sweepings in the trash or back on the lawn.

- Be aware of automobiles leaking fluids.  Use absorbents such as cat litter to soak up drippings – dispose of properly.

- Re-vegetate disturbed and bare areas to maintain vegetative stabilization.

- Do not store materials outdoors (including landscaping materials) unless properly protected from runoff.

## Section 2.4:  Access and Easements

All stormwater management facilities located on the site shall have both a designated access location as well as a maintenance easement.  Refer to the Maintenance Site Plan located in Appendix C (verify this location and name of plan is correct) for access and easement locations.

## Section 2.5:  Safety

Always keep safety considerations at the forefront of inspection procedures.  Hazards should be anticipated and avoided.  Never enter a confined space (outlet structure, manhole, etc.) without proper training or equipment.  A confined space should never be entered without at least one additional person present.

If a toxic or flammable substance is discovered, leave the immediate area and call 911.

Potentially dangerous (i.e. fuel, chemicals, hazardous materials) substances found in the area must be referred to the local Sheriff’s Office immediately for response by the Hazardous Materials Unit.  The emergency contact number is 911.

Vertical drops may be encountered in areas located within and around the facility.  Avoid walking on top of retaining walls or other structures that have a significant vertical drop.  If a vertical drop is identified within the facility that is greater than 48” in height, make the appropriate note/comment on the maintenance inspection form.

If any hazard is found within the facility area that poses an immediate threat to public safety, contact the Sheriff’s Office immediately.

## Section 2.6:  Field Inspection Equipment

It is imperative that the appropriate equipment is taken to the field with the inspector(s).  This is to ensure the safety of the inspector and allow the inspections to be performed as efficiently as possible.  Below is a list of the equipment that may be necessary to perform the inspections of all Permanent Stormwater Control Measure Facilities:

- Protective clothing and boots

- Safety equipment (vest, hard hat, confined space entry equipment)

- Communication equipment

- Operation and Maintenance Manual for the site including stormwater management facility location maps

- Clipboard

- Inspection checklist and form (See Appendix B (verify this location is correct))

- Manhole Lid Remover

- Shovel

Some of the items identified above need not be carried by the inspector (manhole lid remover, shovel, and confined space entry equipment).  However, this equipment should be available in the vehicle driven to the site.

## Section 2.7:  Inspecting Permanent Stormwater Control Measure Facilities

The quality of stormwater entering the waters of the state relies heavily on the proper operation and maintenance of permanent stormwater control measures.  The facilities must be periodically inspected to ensure that they function as designed.  The inspection will determine the appropriate maintenance that is required for the facility.

- Inspection Procedures

  - All stormwater management facilities should be inspected at a minimum of once per year.  Inspections should follow the Standard Operating Procedures in Section 3.

- Inspection Report

  - The person(s) conducting the inspection activities should complete the PSCM (Permanent Stormwater Control Measure) Owner Annual Inspection Checklist and Maintenance Reporting Form located in Appendix B (verify this location is correct).

- Answering “Yes” to any of the questions indicates that corrective action shall be taken.

- Inspection Summary/Additional Comments: Additional explanations to inspection items, and observations about the facility not covered by the form should be recorded.

- The person(s) inspecting the site and the property owner shall sign the inspection form verifying the inspection and required maintenance has been completed.  The form should be provided to PCPW on an annual basis (no later than April 1st of each year).

## Section 2.8:  Maintaining Stormwater Management Facilities

Stormwater management facilities must be properly maintained to ensure that they operate correctly and provide the water quality treatment for which they were designed.  Routine maintenance performed on a frequently scheduled basis can help avoid more costly rehabilitative maintenance that results when facilities are not adequately maintained.

- Maintenance Categories

Stormwater management facility maintenance programs are separated into three broad categories of work.  These categories are based largely on the Urban Drainage and Flood Control District's Maintenance Program for regional drainage facilities.  The categories are separated based upon the magnitude and type of the maintenance activities performed.  A description of each category follows:

  - Routine Work

The majority of this work consists of scheduled mowing and trash and debris pickups for stormwater management facilities during the growing season.  This includes items such as the removal of debris/material that may be clogging the outlet structure well screens and trash racks.  It also includes activities such as weed control, mosquito treatment, and algae treatment. These activities normally will be performed numerous times during the year.  These items can be completed without any prior correspondence with PCPW; however, completed inspection and maintenance forms should be submitted to PCPW for each inspection and maintenance activity.

  - Restoration Work

This work consists of a variety of isolated or small-scale maintenance and work needed to address operational problems.  Most of this work can be completed by a small crew, with minor tools, and small equipment.  These items require prior correspondence with PCPW and require that completed maintenance forms be submitted to PCPW for each maintenance activity.

  - Rehabilitation Work

This work consists of large-scale maintenance and major improvements needed to address failures within the stormwater management facilities.  This work requires consultation with PCPW and may require an engineering design with construction plans to be prepared for review and approval.  This work may also require more specialized maintenance equipment, surveying, construction permits, or assistance through private contractors and consultants.  These items require prior correspondence with PCPW and require that completed maintenance forms be submitted to PCPW for each maintenance activity.

- Maintenance Personnel

Maintenance personnel must be qualified to properly maintain stormwater management facilities.  Inadequately trained personnel can cause additional problems resulting in additional maintenance costs.

- Maintenance Forms

The PSCM (Permanent Stormwater Control Measure) Owner Annual Inspection Checklist and Maintenance Reporting Form provides a record of maintenance activities.  Maintenance Reporting Forms should be completed by the contractor or property owner completing the required maintenance items.  If the contractor completes the form, it should then be reviewed by the property owner or an authorized agent of the property owner and submitted on an annual basis (no later than April 1st) to the PCPW.

Refer to Section 2.2 of this Manual regarding the annual reporting of inspections and maintenance activities performed.

# Section 3:  Standard Operating Procedures for Inspection and Maintenance for Detention Basins (PSCMs)

This section documents standard operating procedures for inspection and maintenance for detention basins.

## Section 3.1:  Background

Detention Basins (PSCMs) are one of the most common types of Stormwater Management Facilities utilized within Pueblo County.  A PSCM is a basin designed to be used for flood control by detaining developed stormwater flows and releasing them at historical runoff rates.

PSCMs facilitate sediment removal to improve water quality.  As the PSCM stores stormwater for a brief period of time, a process called sedimentation occurs.  This allows sediment to drop out of the stormwater prior to passing through the outlet structure.

The storage of materials that decrease the volume of stormwater detention or may impact water quality shall not be located within the limits of the PSCMs.

## Section 3.2:  Inspecting Detention Basins (PSCMs)

PSCMs have several features that are designed to serve a particular function.  Many times, the proper function of one feature depends on another.  For example, if the collection and removal of sediment is not properly maintained, it could negatively affect the performance of a feature downstream (outlet structure, etc.).

- Access and Easements

  - Inspection or maintenance personnel may utilize the Maintenance Site Plan located in Appendix C (verify this location and name of plan is correct).  The map contains the location(s) of the access points and maintenance easements of the PSCM(s) within the development.

- Stormwater Management Facility Locations

  - Inspection or maintenance personnel may utilize the Maintenance Site Plan located in Appendix C (verify this location and name of plan is correct)), containing the location(s) of the PSCM(s) within this development.

- Detention Basin (PSCM) Features

- It is critical that each feature of the PSCM is properly inspected and maintained to ensure that the overall facility functions as it was intended.  Below is a list and description of the most common features within a PSCM and the corresponding maintenance inspection items that can be anticipated:

TABLE 1

Most Common Features & Maintenance within a PSCM

| PSCM Features | Sediment Removal | Mowing/ Weed Control | Trash & Debris Removal | Erosion | Overgrown Vegetation Removal | Standing Water (mosquito/ algae control) | Structure Repair |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inflow Points (outfalls) | X |  | X |  |  |  | X |
| Low flow channel | X |  | X | X | X |  | X |
| Bottom Stage | X | X | X | X | X | X |  |
| Outlet Works | X |  | X | X |  |  | X |
| Emergency Spillway |  |  | X | X | X |  | X |
| Upper Stage |  |  | X | X |  |  |  |
| Embankment |  | X |  | X | X |  |  |

- Inflow Points (Outfalls)

Inflow Points or Outfalls into PSCMs are the point source of the stormwater discharge into the facility.  An inflow point is commonly a storm sewer pipe with a flared end section that discharges into the PSCM.  In some instances, an inflow point could be a drainage channel or ditch that flows into the facility.

An energy dissipater (riprap or hard armor protection) is typically immediately downstream of the discharge point into the PSCM to protect from erosion.  In some cases, the storm sewer outfall can have a toe-wall or cut­ off wall immediately below the structure to prevent undercutting of the outfall from erosion.

The typical maintenance items that are found with inflow points are as follows:

a.	Riprap Displaced – Many times, because the repeated impact/force of water, the riprap can shift and settle.  If any portion of the riprap apron appears to have settled, soil is present between the riprap, or the riprap has shifted, maintenance may be required to ensure future erosion is prevented.

b.	Erosion Present/Outfall Undercut – In some situations, the energy dissipater may not have been sized, constructed, or maintained appropriately and erosion has occurred.  Any erosion within the vicinity of the inflow point will require maintenance to prevent damage to the structure(s) and sediment transport within the facility.

c.	Sediment Accumulation – Because of the turbulence in the water created by the energy dissipater, sediment often deposits immediately downstream of the inflow point.  To prevent a loss in hydraulic performance of the upstream infrastructure, sediment that accumulates in this area must be removed in a timely manner.

d.	Structural Damage – Structural damage can occur at any time during the life of the facility.  Typically, for an inflow, the structural damage occurs to the pipe flared-end-section (concrete or steel).  Structural damage can lead to additional operating problems with the facility, including loss of hydraulic performance.

e.	Woody Growth/Weeds Present – Undesirable vegetation can grow in and around the inflow area to a PSCM that can significantly affect the performance of the drainage facilities discharging into the facility.  This type of vegetation includes trees, brush, and weeds. If this vegetation is not routinely mowed/removed, the growth can cause debris/sediment to accumulate, resulting in blockage of the discharge.  Also, tree roots can cause damage to the structural components of the inflow.  Routine maintenance is essential for trees (removing a small tree/sapling is much cheaper and "quieter" than a mature tree).  In addition, noxious weeds growing in the facility can result in the loss of desirable native vegetation and impact adjacent open spaces/land.

- Low-flow Channel (Trickle Channel)

The trickle channel conveys stormwater from the inflow points of the PSCM to the outlet structure.  The trickle channel is typically made of concrete.  However, grass lined (riprap sides protected) is also common and can provide for an additional means of water quality within the PSCM.

The typical maintenance items that are found with trickle channels are as follows:

a.	Sediment/Debris Accumulation – Trickle channels are typically designed with a relatively flat slope that can promote sedimentation and the collection of debris.  Also, if a trickle channel is grass lined it can accumulate sediment and debris at a much quicker rate.  Routine removal of accumulated sediment and debris is essential in preventing flows from circumventing the trickle channel and affecting the dry storage portion of the pond.

b.	Concrete/Riprap Damage – Concrete can crack, spall, and settle and must be repaired to ensure proper function of the trickle channel.  Riprap can also shift over time and must be replaced/repaired as necessary.

c.	Woody Growth/Weeds Present – Because of the constant moisture in the area surrounding the trickle channel, trees, brush and weeds can become a problem.  This type of vegetation can affect the capacity of the trickle channel and can allow flows to circumvent the feature.

- Erosion Outside of Channel – In larger precipitation events, the trickle channel capacity will likely be exceeded.  This can result in erosion immediately adjacent to the trickle channel and must be repaired to prevent further damage to the structural components of the PSCM.

iii.	Bottom Stage

The bottom stage is lower than the upper stage and is located in front of the outlet works structure.  The bottom stage is designed to store the smaller runoff events, assists in keeping the majority of the basin bottom dry, resulting in easier maintenance operations, and enhances the facilities pollutant removal capabilities.  This area of the PSCM may develop wetland vegetation.

The typical maintenance items that are found with the bottom stage are as follows:

- Sediment/Debris Accumulation – The bottom stage can frequently accumulate sediment and debris.  This material must be removed to maintain pond volume and proper function of the outlet structure.

- Woody Growth/Weeds Present – Because of the constant moisture in the soil surrounding the bottom stage, trees, brush and weeds can create operational problems for the PSCM.  If this vegetation is not routinely mowed/removed, the growth can cause debris/sediment to accumulate, which can cause problems with other PSCM features.  Also, tree roots can cause damage to the structural components of the outlet works.  Routine management is essential for trees (removing a small tree/sapling is much cheaper and "quieter" than a mature tree).

- Outlet Works

The outlet works is the feature that drains the PSCM at specified rates. The outlet works is typically constructed of reinforced concrete into the embankment of the PSCM. The concrete structure typically has steel orifice plates anchored/embedded into it to control stormwater release rates. The larger openings (flood control) on the outlet structure typically have trash racks over them to prevent clogging. The water quality orifice plate (smaller diameter holes) will typically have a well screen covering it to prevent smaller materials from clogging it. The outlet structure is the single most important feature in the PSCM operation. Proper inspection and maintenance of the outlet works is essential in ensuring the long-term operation of the PSCM.

The typical maintenance items that are found with the outlet works are as follows:

a.	Trash Rack/Well Screen Clogged - Floatable material that enters the PSCM will most likely make its way to the outlet structure.  This material is trapped against the trash racks and well screens on the outlet structure (which is why the rack/screens are there).  This material must be removed on a routine basis to ensure the outlet structure drains in the specified design period.

b.	Structural Damage – The outlet structure is primarily constructed of concrete, which can crack, spall, and settle.  The steel trash racks and well screens are also susceptible to damage.

c.	Orifice Plate Missing/Not Secure – Many times residents, property owners, or maintenance personnel will remove or loosen orifice plates if they believe the pond is not draining properly.  Any modification to the orifice plate(s) will significantly affect the designed discharge rates for water quality and/or flood control.  Modification of the orifice plates is not allowed without approval from PCPW.

d.	Manhole Access – Access to the outlet structure is necessary to properly inspect and maintain the facility.  If access is difficult or not available to inspect the structure, chances are it will be difficult to maintain as well.

e.	Trees/Brush/Weeds Present – Because of the constant moisture in the soil surrounding the outlet works, trees, brush and weeds can create operational problems for the PSCM.  If this is not routinely mowed/removed, the growth can cause debris/sediment to accumulate around the outlet works, which can cause problems with other PSCM features.  Also, tree roots can cause damage to the structural components of the outlet works.  Routine management is essential for trees (removing a small tree/sapling is much cheaper and "quieter" than a mature tree).

v.	Emergency Spillway

An emergency spillway is typical of all PSCMs and designed to serve as the overflow in the event the volume of the pond is exceeded.  The emergency spillway is typically armored with riprap (or other hard armor) and is sometimes buried with soil.  The emergency spillway is typically a weir (notch) in the pond embankment.  Proper function of the emergency spillway is essential to ensure flooding does not affect adjacent properties.

The typical maintenance items that are found with emergency spillways are as follows:

a.	Riprap Displaced – As mentioned before, the emergency spillway is typically armored with riprap to provide erosion protection.  Over the life of a PSCM, the riprap may shift or dislodge due to flow.

b.	Erosion Present – Although the spillway is typically armored, stormwater flowing through the spillway can cause erosion damage.  Erosion must be repaired to ensure the integrity of the basin embankment and proper function of the spillway.

c.	Woody Growth/Weeds Present – Management of trees, brush, and weeds is essential in the proper long-term function of the spillway.  Larger trees or dense shrubs can capture larger debris entering the PSCM and reduce the capacity of the spillway.

d.	Obstruction Debris – The spillway must be cleared of any obstruction (man-made or natural) to ensure the proper design capacity.

- Upper Stage (Dry Storage)

The upper stage of the PSCM provides the majority of the water quality flood detention volume.  The upper stage is the largest feature/area of the basin.  Sometimes, the upper stage can be utilized for park space and other approved uses in larger PSCMs.  With proper maintenance of the bottom stage, the upper stage should not experience much sedimentation; however, bottom elevations should be monitored to ensure adequate volume.

The typical maintenance items that are found with upper stages are as follows:

a.	Vegetation Sparse – The upper basin is the most visible part of the PSCM, and therefore aesthetics is important.  Adequate and properly maintained vegetation can greatly increase the overall appearance and acceptance of the PSCM by the public.  In addition, vegetation can reduce the potential for erosion and subsequent sediment transport to the other areas of the pond.

b.	Trees/Undesirable Vegetation – Although some trees and vegetation may be acceptable in the upper basin, some thinning of cottonwoods and willows may be necessary.  Remember, the basin will have to be dredged to ensure volume, and large trees and shrubs will be difficult to protect during that operation.

c.	Standing Water/Boggy Areas – Standing water or boggy areas in the upper stage is typically a sign that some other feature in the pond is not functioning properly.  Routine maintenance (mowing, trash removal, etc.) can be extremely difficult for the upper stage if the ground is saturated.  If this inspection item is checked, make sure you have identified the root cause of the problem.

d.	Sediment Accumulation – Although other features within the PSCM are designed to capture sediment, the upper storage area will collect sediment over time.  Excessive amounts of sedimentation will result in a loss of storage volume.  It may be more difficult to determine if this area has accumulated sediment without conducting a field survey.

Below is a list of indicators:

- Ground adjacent to the trickle channel appears to be several inches higher than concrete/riprap

- Standing water or boggy areas in upper stage

- Uneven grades or mounds

- Bottom stage has excessive amounts of sediment

e.	Erosion (banks and bottom) – The bottom grades of the dry storage are typically flat enough that erosion should not occur.  However, inadequate vegetative cover may result in erosion of the upper stage.  Erosion that occurs in the upper stage can result in increased dredging/maintenance of the bottom stage.

f.	Trash/Debris – Trash and debris can accumulate in the upper area after large events, or from illegal dumping.  Over time, this material can accumulate and clog the PSCM outlet works.

g.	Maintenance Access – Most PSCMs typically have a maintenance access path to the upper stage.  This access path should be inspected to ensure the surface is still drivable.  Some of the smaller PSCMs may not have maintenance access paths; however, the inspector should verify that access is available from adjacent properties.

vii.	Miscellaneous/Other

There are a variety of inspection/maintenance issues that may not be attributed to a single feature within the PSCM.  This category on the reporting form is for maintenance items that are commonly found in the PSCM but may not be attributed to an individual feature.

a.	Encroachment in Easement Area – Private lots/property can sometimes be located very close to the PSCMs, even though they are required to be located within drainage easements.  Property owners may place landscaping, fencing, or other items within the easement area that may affect maintenance or the operation of the facility.  If these items have the potential to affect the operation, volume or water quality of the facility, they should be removed.

b.	Graffiti/Vandalism – Damage to the PSCM infrastructure can be caused by vandals.  If criminal mischief is evident, the inspector should forward this information to the local Sheriff's Office.

c.	Public Hazards – Public hazards include items such as vertical drops of greater than 4-feet, containers of unknown/suspicious substances, exposed metal/jagged concrete on structures.  If any hazard is found within the facility area that poses an immediate threat to public safety, contact the local Sheriff at 911 immediately!

d.	Burrowing Animals/Pests – Prairie dogs and other burrowing rodents may cause damage to the PSCM features and negatively affect the vegetation within the PSCM.

e.	Other – Any miscellaneous inspection/maintenance items not contained on the form should be entered here.

## Section 3.3:  Inspection Forms

The PSCM Owner Annual Inspection Checklist and Maintenance Reporting Form is located in Appendix B (verify this location is correct).  Inspection forms should be completed by the person(s) conducting the inspection activities.  Each form should be reviewed and submitted by the property owner or property manager to PCPW per the requirements of the Operations and Maintenance Manual.  Any inspection forms shall be kept indefinitely and made available to PCPW upon request.

## Section 3.4:  Maintaining Detention Basins (PSCMs)

PSCMs require routine maintenance to ensure they function as designed.

            - Maintenance Personnel

Maintenance personnel must be qualified to properly maintain PSCMs.  Inadequately trained personnel can cause additional problems resulting in additional maintenance costs.

            - Equipment

It is imperative that the appropriate equipment and tools are taken to the field with the operations crew.  The types of equipment/tools will vary depending on the task at hand.  Below is a list of tools, equipment, and material(s) that may be necessary to perform maintenance on a PSCM:

                - Loppers/Tree Trimming Tools

                - Mowing Tractors

                - Trimmers (extra string)

                - Shovels/Rakes

                - All Surface Vehicle (ASVs)

                - Skid Steer

                - Backhoe

                - Long-reach Excavator

                - Dump Truck

                - Jet-Vac Machine

                - Engineers Level (laser)

                - Riprap (Minimum - Type M)

                - Filter Fabric

                - Erosion Control Blanket(s)

                - Seed Mix (Native)

                - Illicit Discharge Cleanup Kits

                - Trash Bags

                - Tool (wrenches, screw drivers, hammers , etc.)

                - Chain Saw

                - Confined Space Entry Equipment

                - Approved Stormwater Facility Operation and Maintenance Manual

Some of the items identified above may not be needed for every maintenance operation.  However, this equipment should be available to the maintenance operations crews should the need arise.

3.	Safety

Vertical drops may be encountered in areas located within and around the facility.  Avoid walking on top of retaining walls or other structures that have a significant vertical drop.  If a vertical drop is identified within the PSCM that is greater than 48" in height, make the appropriate note/comment on the maintenance inspection form.

- Maintenance Forms

The PSCM Maintenance Form provides a record of each maintenance operation performed by maintenance contractors.  The PSCM Maintenance Form should be filled out in the field after the completion of the maintenance operation.  Each form should be reviewed and submitted by the property owner or property manager to the PCPW per the requirements of the Operations and Maintenance Manual.  The PSCM Owner Inspection Checklist and Maintenance form is located in Appendix B (verify this location is correct).

- Maintenance Categories and Activities

A typical PSCM Maintenance Program will consist of three broad categories of work.  Within each category of work, a variety of maintenance activities can be performed on a PSCM.  A maintenance activity can be specific to each feature within the PSCM, or general to the overall facility.  This section of the SOP explains each of the categories and briefly describes the typical maintenance activities for a PSCM.

A variety of maintenance activities are typical of PSCMs.  The maintenance activities range in magnitude from routine trash pickup to the reconstruction of drainage infrastructure.

- Routine Maintenance Activities

The majority of this work consists of regularly scheduled mowing and trash and debris pickups for stormwater management facilities during the growing season.  This includes items such as the removal of debris/material that may be clogging the outlet structure well screens and trash racks.  It also includes activities such as weed control, mosquito treatment, and algae treatment.  These activities normally will be performed numerous times during the year and can be completed without any prior correspondence with PCPW; however, completed inspection and maintenance forms should be submitted to PCPW for each inspection and maintenance activity.

The Maintenance Activities are summarized below in Table 2, and further described in the following sections.

TABLE 2

Summary of Routine Maintenance Activities

| MAINTENANCE ACTIVITY | MINIMUM FREQUENCY | LOOK FOR: | MAINTENANCE ACTION |
| --- | --- | --- | --- |
| Trash/Debris Removal | Twice annually | Trash and debris in PSCM | Remove and dispose of trash and debris |
| Mowing | Routine – Twice annually or depending on aesthetics | Excessive grass height/aesthetics | Mow grass to a height of 4” to 6” |
| Outlet Works Cleaning | As needed – after significant rain events – twice annually minimum | Clogged outlet structure; ponding water | Remove and dispose of debris/trash/sediment to allow outlet to function properly |
| Weed Control | Minimum twice annually | Noxious weeds; unwanted vegetation | Treat with herbicide or hand pull; Consult the local weed specialist |
| Mosquito Treatment | As needed | Standing water/mosquito habitat | Treat with EPA approved chemicals* |
| Algae Treatment | As needed | Standing water/Algae growth/green color | Treat with EPA approved chemicals |

- Trash/Debris Removal

Trash and debris must be removed from the entire PSCM area to minimize outlet clogging and to improve aesthetics.  This activity must be performed prior to mowing operations.

Frequency – Routine.  Prior to mowing operations and at a minimum of twice annually

- Mowing

Occasional mowing is necessary to limit unwanted vegetation and to improve the overall appearance of the PSCM.

Frequency – Routine.  Minimum of twice annually or depending on aesthetics.

- Outlet Works Cleaning

Debris and other materials can clog the outlet work's well screen, orifice plate(s) and trash rack.  This activity must be performed anytime other maintenance activities are conducted to ensure proper operation.

Frequency – Routine.  After significant rainfall event or concurrently with other maintenance activities.

- Weed Control

Noxious weeds and other unwanted vegetation must be treated as needed throughout the PSCM.  This activity can be performed either through mechanical means (mowing/pulling) or with herbicide.  Consultation with the local agencies is highly recommended prior to the use of herbicide.

Frequency – As needed.

- Mosquito/Algae Treatment

Treatment of permanent pools is necessary to control mosquitoes and undesirable aquatic vegetation that can create nuisances.  Only EPA approved chemicals/materials can be used in areas that are warranted.

- Minor Maintenance Activities

This work consists of a variety of isolated or small-scale maintenance or operational problems.  Most of this work can be completed by a small crew, tools, and small equipment.  These items require completed inspection and maintenance forms to be submitted to PCPW for each inspection and maintenance activity.

TABLE 3

Summary of Minor Maintenance Activities

| MAINTENANCE ACTIVITY | MINIMUM FREQUENCY | LOOK FOR: | MAINTENANCE ACTION |
| --- | --- | --- | --- |
| Sediment Removal | As needed, typically every 1-2 years | Sediment build-up; decrease in pond volume | Remove and dispose of sediment |
| Erosion Repair | As needed, based upon inspection | Rills/gullies forming on side slopes, trickle channel, or other areas | Repair eroded areas; revegetate; address source of erosion |
| Vegetation Removal/ Tree Thinning | As needed, based upon inspection | Large trees/wood vegetation in lower chamber of pond | Remove vegetation; restore grade and surface |
| Drain Cleaning/Jet Vac | As needed, based upon inspection | Sediment build-up/non-draining system | Clean drains; Jet Vac if needed |

- Sediment Removal

Sediment removal is necessary to maintain the original design volume of the PSCM and to ensure proper function of the infrastructure.  Regular sediment removal (minor) from the bottom stage, inflow(s), and trickle channel can significantly reduce the frequency of major sediment removal activities (dredging) in the upper and lower stages.  The minor sediment removal activities can typically be addressed with shovels and smaller equipment.  Major sediment removal activities will require larger and more specialized equipment.

Stormwater sediments removed from PSCMs do not meet the criteria of "hazardous waste". However, these sediments are contaminated with a wide array of organic and inorganic pollutants and handling must be done with care.  Sediments from permanent pools must be carefully removed to minimize turbidity, further sedimentation, or other adverse water quality impacts.  Sediments should be transported by motor vehicle only after they are dewatered.  All sediments must be taken to a landfill for proper disposal.  Prompt and thorough cleanup is important should a spill occur during transportation.

Frequency – Nonroutine.  As necessary based upon inspections.  Sediment removal in the forebay and trickle channel may be necessary as frequently as every 1-2 years.

- Erosion Repair

The repair of the eroded areas is necessary to ensure the proper function of the PSCM, minimize sediment transport, and to reduce potential impacts to other features.  Erosion can vary in magnitude from minor repairs to trickle channels, energy dissipaters, and rilling to major gullies in the embankments and spillways.  The repair of eroded areas may require the use of excavators, earthmoving equipment, riprap, concrete, erosion control blankets, and turf reinforcement mats.

Frequency – Nonroutine.  As necessary based upon inspections.

- Vegetation Removal/Tree Thinning

Dense stands of trees, brush and weeds can create maintenance problems for the infrastructure within a PSCM.  Tree roots can damage structures and invade pipes/channels thereby blocking flows.  Also, trees growing in the upper and lower stages of the PSCM will most likely have to be removed when sediment/dredging operations occur.  A small tree is easier to remove than a large tree, therefore, regular removal/thinning is imperative.  All trees and woody vegetation that is growing in the bottom of the PSCM or near structures (inflows, trickle channels, outlet works, emergency spillways, etc.) should be removed.  Any trees or woody vegetation in the PSCM should be limited to the upper portions of the pond banks.

Frequency – Nonroutine.  As necessary based upon inspections.

- Clearing Drains/Jet-Vac

A PSCM may contain many structures, openings, and pipes that can be frequently clogged with debris.  These blockages can result in a decrease of hydraulic capacity and create standing water in areas outside of the micro­ pool.  Many times, the blockage to this infrastructure can be difficult to access and/or clean.  Specialized equipment (Jet-vac machines) may be necessary to clear debris from these difficult areas.

Frequency – Nonroutine.  As necessary based upon inspections.

- Major Maintenance Activities

This work consists of larger maintenance/operational problems and failures within the stormwater management facilities.  All of this work requires consultation with PCPW to ensure the proper maintenance is performed.  This work requires that the engineering staff review the original design and construction drawings to access the situation and assign the necessary maintenance.  This work may also require more specialized maintenance equipment, design/details, surveying, or assistance through private contractors and consultants.

TABLE 4

Summary of Major Maintenance Activities

| MAINTENANCE ACTIVITY | MINIMUM FREQUENCY | LOOK FOR: | MAINTENANCE ACTION |
| --- | --- | --- | --- |
| Major Sediment Removal | As needed based upon scheduled inspection | Large quantities of sediment, reduced pond capacity | Remove and dispose of sediment. Repair vegetation as needed. |
| Major Erosion Repair | As needed based upon scheduled inspection | Severe erosion including gullies, excessive soil displacement, areas of settlement holes | Repair erosion – find cause of the problem and address to avoid future erosion |
| Structural Repair | As needed based upon scheduled inspection | Deterioration and/or damage to structural components – broken concrete, damaged pipes, outlet works | Structural repair to restore the structure to its original design |

- Major Sediment Removal

This work consists of removal of large quantities of sediment or removal of sediment from vegetated areas.  Care shall be given when removing large quantities of sediment and sediment deposited in vegetated areas.  Large quantities of sediment need to be carefully removed, transported and disposed of.  Vegetated areas need special care to ensure design volumes and grades are preserved.

Frequency – Nonroutine.  Repair as needed based upon inspections.

- Major Erosion Repair

This work consists of filling and revegetating areas of severe erosion.  Determining the cause of the erosion, as well as correcting the condition that caused the erosion, should also be part of the erosion repair.  Care should be given to ensure design grades and volumes are preserved.

Frequency – Nonroutine.  Repair as needed based upon inspections.

- Structural Repair

A PSCM includes a variety of structures that can deteriorate or be damaged during routine maintenance.  These structures are constructed of steel and concrete that can degrade or be damaged and may need to be repaired or re-constructed from time to time.  These structures include items like outlet works, trickle channels, forebays, inflows and other features.  In­ house operations staff can perform some of the minor structural repairs.

Major repairs to structures may require input from a structural engineer and specialized contractors.  Consultation with PCPW Engineering Staff should take place prior to all structural repairs.

Frequency – Nonroutine.  Repair as needed based upon inspections.

# Appendix A:  Stormwater Facility Maintenance Agreement(s) – template (Remove sheets and replace with signed a notarized agreement

.

# Appendix B:  PSCM Owner Inspection Checklist and Maintenance Reporting Form

# Appendix C:  Maintenance Site Plan – change appendix name to reflect sheet name

The site drainage plan, stormwater facility plan, as well as any applicable plans/maps should be inserted in this Appendix. (this sheet shall be removed prior to submittal).